Storage format of the morphological data

The morphological data required for the HCM programs is provided by the TWG HCM SWG Program (Technical Working Group Harmonized Calculation Method Sub Working Group Program) and is available at the internet site of the Vienna Agreement (http://va.bmwi.bund.de).

The data is elaborated using GTOPO30 data.

The morphological database is a raster database with the same grid and structure as the terrain height database.

Each entry consists of two bytes, one for the predominant height of the surface (trees, buildings) and one for the class of the morphology information.

Because each grid point represents an area of 3 x 3 (3 x 6) seconds, more than one class of morphology is possible, e.g. a part is buildings, an other part is trees. In those cases there are different heights for this area. It is possible, to define more than one class, but only one height.

The height information is one byte. Therefore it is possible to define heights from 0 m to

255 m. The height is the predominant height of the area represented by this grid point, e.g. if there are 70 % buildings with 10 m height, 20% trees with 12 m height and 10% roads with 0 m height, 10 m is taken to represent this area.

The class of morphology consists of one byte. Therefore 8 different classes (bits) are possible. For the fixed service land, sea and coastal area are required, for the mobile service only land and sea are required.

- all bits are 0			normal land

- bit 0 is 1			sea, ocean

- bit 1 is 1			small lake, river, small portions of water (no sea, no ocean!)

- bit 2 is 1			coastal area

- bit 3 is 1			villages, towns (buildings)

- bit 4 is 1			trees

- bits 5 to 7			for future use

In general, a morphological database is not required for all countries applying the HCM software. If there is no sea or coastal area (e.g. Austria, Slovakia), the use of a morphological database is not mandatory.

The morphological database offered by the TWG HCM SWG Program does not have height information (all heights are 0 m). Only the morphological classes ‘normal land’, ‘sea / ocean’ and ‘costal area’ are supplied.

All morphological data consists of 2 bytes. The first byte represents the class of morphology, the second byte is the height information.

- Morphological data of a 5 x 5 minutes square (approximate 9 x 5 km) is combined in one data-record.

- 12 * 12 records (=144 records;=1 * 1 degree) are stored in one file.

- The filename is (example): E007N50.63M

| where | E007 | = | longitude of the South -West corner, |
| --- | --- | --- | --- |
|  | N50 | = | latitude of the South -West corner, |
|  | 63 | = | resolution in seconds longitude (6) and latitude (3) and |
|  | M | = | Morphological data (E for elevation data) |

- Position of records inside the file:

- North

|  | 133 | 134 | 135 | 136 | 137 | 138 | 139 | 140 | 141 | 142 | 143 | 144 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 121 |  |  |  |  |  |  |  |  |  |  | 132 |  |
|  | 109 |  |  |  |  |  |  |  |  |  |  | 120 |  |
|  | 97 |  |  |  |  |  |  |  |  |  |  | 108 |  |
|  | 85 |  |  |  |  |  |  |  |  |  |  | 96 |  |
|  | 73 |  |  |  |  |  |  |  |  |  |  | 84 |  |
| West | 61 |  |  |  |  |  |  |  |  |  |  | 72 | East |
|  | 49 |  |  |  |  |  |  |  |  |  |  | 60 |  |
|  | 37 |  |  |  |  |  |  |  |  |  |  | 48 |  |
|  | 25 |  |  |  |  |  |  |  |  |  |  | 36 |  |
|  | 13 |  |  |  |  |  |  |  |  |  |  | 24 |  |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |  |

South

All files with the same longitude are stored in the same subdirectory. The name of the subdirectory is equal to the first four characters of the filename (example: E007).

All subdirectories are stored in the (top level) directory MORPHO. A valid filename with the full path therefore is:

C:\MORPHO\E010\E010N45.33M

Note: Only the drive letter ‘C’ may be replaced by any other valid drive letter. The rest of the directory structure is fixed. This fixed directory structure is build in the HCM programs. Only the drive letter is an input value to the HCM programs. The rest of the filename / subdirectory name is calculated inside the HCM programs taking into account the co-ordinates.

In Europe, a 5 x 5 minutes square contains north of 50 degrees latitude 101 x 51 values = 5.151 values and south of 50 degrees latitude 101 x 101 values = 10.201 values. One value = 2 bytes. The length of data records therefore is 10.302 or 20.402 bytes.

Record description:

- Length: fixed, 10.302 or 20.402 Bytes

No carriage control!

- The data inside the record is combined from East to West and from South to North.

Example:  (South of 50 degrees latitude, resolution in East-West-direction = 3 seconds; number of morphological data)

|  |  |  |  | North |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10101 | 10102 | 10103 |  |  |  |  | 10200 |  | 10201 |  |
|  | 10000 | 10001 | 10002 |  |  |  | 10099 |  | 10100 |  |  |
| West |  |  |  |  |  |  |  |  |  |  | East |
|  | 102 | 103 | 104 |  |  |  | 201 |  | 202 |  |  |
|  | 1 | 2 | 3 |  |  |  |  | 100 |  | 101 |  |

- South