# Interface to the HCMMS_V7 subroutine in FORTRAN 90.

The HCMMS_V7 program is only a subroutine. To run the program, a surrounding program is required.

The subroutine HCMMS_V7 is written in FORTRAN 90 language (with Compaq Visual Fortran Professional Edition 6.6B). A simple surrounding program is listed below. A more complex surrounding program is supplied by the HCM SWG Program and can be found on the Internet site of the Agreement.

!

!	Simple surrounding program for the HCMMS_V7 subroutine.

!

PROGRAM Test

!

IMPLICIT	NONE

INCLUDE	‘HCM_MS_V7_definitions.f90’

!

!	Prepare all input data

!

CALL HCMMS_V7

!

!	Show all output data

!

END PROGRAM Test

!

All input- and output data is defined in the file ‘HCM_MS_V7_definitions.f90’. Therefore no arguments are included in the subroutine call.

Description of all input data:

Name	Data type	Description

C_mode	Integer*4	Calculation mode, possible values are:

9	UMTS / IMT200 point to point calculation

8	Emergency and security services

(380-400Mhz) point to point calculation

7	normal Agreement coverage calculation

(50% time probability)

6	GSM 1800 – GSM 1800 ML

(42 dBµV/m) calculation

5	GSM 1800 – GSM 1800 FB

(38 dBµV/m) calculation

4	ERMES – ERMES (32 dBµV/m) Calculation

3	GSM 900 – NMT calculation

2	GSM 900 –TACS calculation

1	GSM 900 – GSM calculation

0	normal Agreement point to point calculation

-1	Agreement line calculation (h2 = 10m)

-2	GSM 900 line calculation

(19 dBµV/m, h2=3m)

-3	ERMES line calculation (12 dBµV/m)

-4	ERMES line calculation (32 dBµV/m)

-5	ERMES line calculation (52 dBµV/m)

-6	GSM 1800 line calculation (25 dBµV/m)

-7	Emergency and security service

(380-400 MHz) line calculation

-8	UMTS / IMT2000 line calculation

Coo_Tx	Character*15	Tx co-ordinates, format = ‘015E203052N2040’

Coo_Rx	Character*15	Rx co-ordinates, format = ‘015E203052N2040’

only required if C_mode is 0 or positive

H_Tx_ant	Character*4	Tx antenna height in m

H_Rx_ant	Character*4	Rx antenna height in m, only required if

C_mode is 0 or positive

Tx_frequ	Character*12	Tx frequency, format = ‘00147.77000M’

Rx_frequ	Character*12	Rx frequency, format = ‘00147.77000M’,

only required if C_mode is 0 or positive

Rad_of_Tx_serv_area	Character*5	Radius of Tx service area

Rad_of_Rx_serv_area	Character*5	Radius of Rx service area, only required if

C_mode is 0 or positive

H_Tx_input	Character*4	Input value of Tx site height (if available)

Name	Data type	Description

H_Rx_input	Character*4	Input value of Rx site height (if available)

only required if C_mode is 0 or positive

Max_power	Character*6	Maximum radiated power

Type_of_Tx_ant	Character*1	Type of Tx reference antenna (E / I)

Type_of_Rx_ant	Character*1	Type of Rx reference antenna (E / I)

only required if C_mode is 0 or positive

Chan_occup	Character*1	Channel occupation (0 /1)

PD	DoublePrecision	Point distance for the profile

(default value = 0.1 km)

Perm_FS_input	Character*5	Input value of permissible field strength

(if not filled in, the value is taken from the

table in Annex 1)

Max_CBR_D_input	Character*3	Input value of maximum cross-border range

(if not filled in, the value is taken from the

table in Annex 1)

Sea_temperature	Character*1	Sea temperature (C / W)

Topo_path	Character*63	Path of the terrain height data (e.g. ‘C:\TOPO’)

Morpho_path	Character*63	Path of the morphological data

(e.g. ‘C:\MORPHO’)

Border_path	Character*63	Path of the (border-) line data

(e.g. ‘C:\BORDER’)

D_to_border	Integer*4	Distance to border line (for the selection of

the type of line calculation, 0 = calculation

on the borderline, negative value =

calculation of the maximum cross-border

range, positive value x = calculation on the

x-km line

only required if C_mode is 0 or positive

Land_from	Character*3	Country of Tx or country to calculate from

Land_to	Character*3	Country of Rx or country to calculate to

Rx_ant_gain	Character*4	Gain of Rx antenna

only required if C_mode is 0 or positive

Name	Data type	Description

Depol_loss	Character*4	Depolarization loss

only required if C_mode is 0 or positive

Cor_fact_frequ_diff	Character*4	Correction factor according to frequency

Difference, 	only required if C_mode is 0 or

positive, if missing, this value is calculated

Azi_Tx_input	Character*5	Tx azimuth

Azi_Rx_input	Character*5	Rx azimuth, only required if C_mode is 0 or

positive

Ele_Tx_input	Character*5	Tx elevation

Ele_Rx_input	Character*5	Rx elevation, only required if C_mode is 0 or

positive

D_sea_input	Character*5	Input value of distance over sea; if filled, the

calculation of distance over sea is switched off

Ant_typ_H_Tx	Character*7	Horizontal antenna type of Tx

Ant_typ_V_Tx	Character*7	Vertical antenna type of Tx

Ant_typ_H_Rx	Character*7	Horizontal antenna type of Rx, only required if

C_mode is 0 or positive

Ant_typ_V_Rx	Character*7 	Vertical antenna type of Rx, only required if

C_mode is 0 or positive

Desig_of_Tx_emis	Character*9	Designation of emission of Tx

Desig_of_Rx_emis	Character*9 	Designation of emission of Rx, only required

If C_mode is 0 or positive

Description of all output data:

Name	Data type	Description

HCM_error	Integer*4	Error value, see list of HCM_error values

Info(i)	Logical*4(20)	List of Info(i) values, see list of Info values

Calculated_FS	Real	Calculated field strength

Perm_FS	Real	Permissible field strength

Prot_margin	Real	Protection margin

Free_space_FS	Real	Free space field strength

Distance	DoublePrecision	Distance between Tx position and Rx position

D_sea_calculated	DoublePrecision	Calculated distance over sea

(or from input value)

Dir_Tx_Rx	DoublePrecision	Horizontal direction from Tx to Rx

Dir_Rx_Tx	DoublePrecision	Horizontal direction from Rx to Tx

V_angle_Tx_Rx	DoublePrecision	Vertical direction from Tx to Rx

V_angle_Rx_Tx	DoublePrecision	Vertical direction from Rx to Tx

H_diff_angle_Tx_Rx	DoublePrecision	Horizontal difference angle from Tx to Rx

H_diff_angle_Rx_Tx	DoublePrecision	Horizontal difference angle from Rx to Tx

V_diff_angle_Tx_Rx	DoublePrecision	Vertical difference angle from Tx to Rx

V_diff_angle_Rx_Tx	DoublePrecision	Vertical difference angle from Rx to Tx

Delta_frequency	DoublePrecision	Frequency difference in Hz

Heff_Tx	Real	Tx effective antenna height

Heff_Rx	Real	Rx effective antenna height

Heff	Real	Total effective antenna height

Dh	Real	Terrain irregularity

Dh_corr	Real	Correction factor according to the

terrain irregularity

Name	Data type	Description

Tx_TCA	Real	Tx clearance angle

Rx_TCA	Real	Rx clearance angle

Tx_TCA_corr	Real	Correction factor according to the

Tx clearance angle

Rx_TCA_corr	Real	Correction factor according to the

Rx clearance angle

ERP_ref_Tx	Real	ERP of the reference transmitter

Land_FS	Real	Land field strength

Sea_FS	Real	Sea field strength

Tx_ant_corr	Real	Correction factor according to the

Tx antenna type (horizontal and vertical)

Rx_ant_corr	Real	Correction factor according to the

Rx antenna type (horizontal and vertical)

Tx_ant_type_corr	Real	Correction factor according to the

Tx reference antenna type (E / I)

Rx_ant_type_corr	Real	Correction factor according to the

Rx reference antenna type (E / I)

Perm_FS_from_table	Real	Permissible field strength from the

Table in Annex 1

Corr_delta_f	Real	Correction factor according to the

Frequency difference (calculated or from

The input value)

Channel_sp_Tx	Real	Channel spacing of Tx in kHz

Channel_sp_Rx	Real	Channel spacing of Rx in kHz

Power_to_Rx	Real	Power in direction of Rx

CBR_D	Real	Maximum cross-border range in km

(from input value or from table in Annex 1)

Name	Data type	Description

Version	Character*5	HCMMS_V7 version number

Coo_Tx_new	Character*15	Calculated Tx co-ordinates

Coo_Rx_new	Character*15	Calculated Rx co-ordinates (or line co-ordinates)

T_Prof(i)	Integer*2(10002)	Terrain height profile

M_Prof(i)	Integer*2(10002)	Morphological profile

PN	Integer*2	Number of profile points

H_Datab_Tx	Integer*2	Tx site height from terrain database

H_Datab_Rx	Integer*2	Rx site height from terrain database