# Interface to the HCMMS_V7_DLL.DLL

To simplify the use of the HCM software, the HCM team build a dynamic link library HCMMS_V7_DLL.DLL.

A lot of programming languages (e.g. Visual Basic, C++) are able to work with DLL’s.

An example in Visual Basic is available from the HCM team.

There are some restrictions in some programming languages: Boolean - and String variables are difficult to handle and the number of arguments is limited. Therefore some modifications in the interface are required:

- No Boolean variables are passed,

- Only one String is passed.

The Interface to (and from) the DLL is:

There is one Subroutine called HCMMS_V7_DLL.

The arguments are:

C_mode

An input value; the mode of calculation; a 4 byte INTEGER (LONG in VB); permissible values = -8 to +9

Bor_dis

An input value; the distance to the borderline; a 4 byte INTEGER (LONG in VB);

Values: 0 = calculations are performed on the borderline, positive value x = calculations are performed at the x-km line, any negative value = calculation of the maximum cross-border range.

PD

An input value; the distance between two profile points (grid size) in km; an 8 byte DOUBLE PRECISION (DOUBLE in VB); this value has to be set to 0.1 (default value).

Distance

An output value; the distance between transmitter and the receiving point in km; an 8 byte DOUBLE PRECISION (DOUBLE in VB).

H_Datab_Tx

An output value; a 2 byte INTEGER (INTEGER in VB); the height of the transmitter site above sea level from the terrain database in m.

H_Datab_Rx

An output value; a 2 byte INTEGER (INTEGER in VB); the height of the receiver site above sea level from the terrain database in m. This value is only valid if CMODE is positive!

HCM_error

An output value; a 4 byte INTEGER (LONG in VB); the error code. Error codes are listed in the HCM documentation.

An additional error code is generated by the DLL software:

Number:	3000

Description:	The string variable passed to the DLL is too short (less than 432 characters).

Heff

An output value; a 4 byte REAL (SINGLE in VB), the effective antenna height in m used for the calculations according the ITU_R method.

Dh

An output value; a 4 byte REAL (SINGLE in VB), the terrain irregularity in m used for the calculations according the ITU_R method.

Dh_corr

An output value; a 4 byte REAL (SINGLE in VB), the correction factor according to the terrain irregularity in dB used for the calculations according the ITU_R method.

Power_to_Rx

An output value; a 4 byte REAL (SINGLE in VB), the power in the direction of the receiver in dBW.

Free_space_FS

An output value; a 4 byte REAL (SINGLE in VB), the free space field strength in dBµV/m.

Land_FS

An output value; a 4 byte REAL (SINGLE in VB), the land field strength in dBµV/m.

Sea_FS

An output value; a 4 byte REAL (SINGLE in VB), the sea field strength in dBµV/m.

Tx_ant_corr

An output value; a 4 byte REAL (SINGLE in VB), the correction factor in dB according to the transmitter antenna type (horizontal and vertcal).

Tx_ant_type_corr

An output value; a 4 byte REAL (SINGLE in VB), the correction factor in dB according to the transmitter antenna type (‘E’ or ‘I’).

Dir_Tx_Rx

An output value; a DOUBLE PRECISION (DOUBLE in VB), the horizontal direction from the transmitter to the receiver in degrees.

V_angle_Tx_Rx

An output value; a DOUBLE PRECISION (DOUBLE in VB), the vertical direction from the transmitter to the receiver in degrees.

Tx_TCA

An output value; a 4 byte REAL (SINGLE in VB), the transmitter clearance angle in degrees.

Rx_TCA

An output value; a 4 byte REAL (SINGLE in VB), the receiver clearance angle in degrees.

Tx_TCA_corr

An output value; a 4 byte REAL (SINGLE in VB), the correction factor according to the transmitter clearance angle in dB.

Rx_TCA_corr

An output value; a 4 byte REAL (SINGLE in VB), the correction factor according to the receiver clearance angle in dB.

D_sea_calculated

An output value; a DOUBLE PRECISION (DOUBLE in VB), the distance over sea in km taken into account during the calculations (either input value or calculated value).

Rx_ant_corr

An output value; a 4 byte REAL (SINGLE in VB), the correction factor in dB according to the receiver antenna type (horizontal and vertical).

Rx_ant_type_corr

An output value; a 4 byte REAL (SINGLE in VB), the correction factor in dB according to the receiver antenna type (‘E’ or ‘I’).

Delta_frequency

An output value; a DOUBLE PRECISION (DOUBLE in VB), the frequency difference between transmitter - and receiver frequency in Hz.

Corr_delta_f

An output value; a 4 byte REAL (SINGLE in VB), the correction factor according to the frequency difference between transmitter - and receiver frequency in dB.

Calculated_FS

An output value; a 4 byte REAL (SINGLE in VB), the calculated field strength in dBµV/m.

Perm_FS

An output value; a 4 byte REAL (SINGLE in VB), the permissible field strength in dBµV/m (input value or calculated value).

CBR_D

An output value; a 4 byte REAL (SINGLE in VB), the maximum cross border range in km (input value or from Agreement).

ERP_ref_Tx

An output value; a 4 byte REAL (SINGLE in VB), the power of the reference transmitter in dBW.

Prot_margin

An output value; a 4 byte REAL (SINGLE in VB), the protection margin in dB (difference of calculated field strength and permissible field strength).

I_str

An input / output value (a part of this string is only input, an other part is only output). A CHARACTER (STRING in VB) variable with at least 432 characters. The content of this string is:

Start position 	Stop position	Description			In-/ Output		Format

1		15	Tx co-ordinates			Input		009E223350N4422

16		30	Rx co-ordinates			Input		009E223350N4422

31		34	Height of Tx site		Input		9999 or four blanks

35		38	Height of Rx site		Input		9999 or four blanks

39		45	Tx hor. ant. Type		Input		000ND00

46		52	Tx vert. ant. type		Input		000ND00

53		57	Tx Azimuth			Input		999.9

58		62	Tx Elevation			Input		+99.9

63		66	Tx Antenna height		Input		9999

67		70	Rx Antenna Height		Input		9999

71		71	Tx Type of antenna		Input		‘E’ or ‘I’

72		77	Max. radiated power		Input		+999.9

78		89	Tx Frequency			Input		99999.99999M

90		90	Channel occupation		Input		‘0’ or ‘1’

91		91	Sea temperature		Input		‘C’ or ‘W’

92		96	Tx Service area			Input		99999

97		101	Rx Service area			Input		99999

102		106	Distance over sea		Input		99999 or 5 blanks

107		118	Rx Frequency			Input		99999.99999M

119		127	Rx Design. of emis.		Input		14k0.....

128		136	Tx Design. of emis.		Input		14k0.....

137		143	Rx hor. ant. Type		Input		000ND00

144		150	Rx vert. ant. Type		Input		000ND00

151		155	Rx Azimuth			Input		999.9

156		160	Rx Elevation			Input		+99.9

161		161	Rx Type of ant.			Input		‘E’ or ‘I’

162		165	Rx Antenna gain		Input		99.9

166		169	Depolarisation loss		Input		99.9

170		174	Perm. Field strength		Input		+99.9 or 5 blanks

175		178	Corr. fact. acc. freq. diff.	Input		99.9 or four blanks

179		181	County code to calcul. to	Input		XXX

182		184	County code of Tx		Input		XXX

185		187	Max. cross border range	Input		999 or 3 blanks

188		250	Path of terrain data		Input		C:\TOPO..... (63 C)

251		313	Path of borderline data		Input		C:\BORDER (63 C)

314		376	Path of morphological data	Input		C:\MORPHO (63 C)

377		382	Version number		Output

383		402	Info values (field, 20 values)	Output		‘T’rue  or ‘F’alse

403		417	Tx co-ordinates calculated	Output		008E213651N2137

418		432	Rx co-ordinates calculated	Output		007E413672N4127

433		???	Debug directory (optional)	Input		e.g. ‘C:\Temp’

Note:

If more than 432 characters are passed, the remaining part (> 432) must be a valid directory name.

If a valid directory is given, the HCMMS_V7_DLL will write two text-files into this directory:

INPUT.TXT and OUTPUT.TXT every time it is called.

INPUT.TXT contains all input values passed to the DLL and OUTPUT.TXT stores all output values coming from the DLL.

# Additional files

To run the DLL on your system, you need some additional run-time files.

The self-installing executable kit ‘VFRUN66BI.exe’ installs the run-time components needed to run Visual Fortran applications on systems, which do not have Visual Fortran, installed.

The components installed by this kit are:

- DFORRT.DLL - Visual Fortran non-threaded run-time support

- DFORMD.DLL - Visual Fortran threaded run-time support

- FQWIN.HLP - QuickWin run-time help file

- MSVCRT.DLL - Microsoft Visual C run-time support

- OLEAUT32.DLL - Microsoft OLE Automation

- OLEPRO32.DLL - Microsoft OLE Automation

- STDOLE32.TLB - Microsoft OLE Automation

Files will be installed only if newer versions do not exist on the system. The files in this kit can be used with earlier versions of Visual Fortran.

The file ‘VFRUN66BI.exe’ is available on the server.