User Guide for the Harmonized Calculation Method on PC

Table of contents

Item	Page

1. 	General	1

2. 	Installation	1

3. 	Calculation modes	3

4. 	Application of cases listed in Annex 5 of the VA01	6

5. 	Input values	8

6. 	Output values	10

Error values 	11

Possible info values 	12

7. 	Using text files for the input	12

Example 1	13

Example 2	14

8. 	Calculations on lines	15

9. 	Questions and answers	16

User Guide for the Harmonized Calculation Method on PC

1. General:

The Harmonized Calculation Method (HCM) is a program to perform calculations between two points or from a point to a line as described in the Annexes of the Vienna Agreement 2001 (VA01). In addition the HCM program may be used to perform calculations for ERMES, GSM900, GSM 1800, the Emergency / Security Services band 380 - 400 MHz and the UMTS/IMT200 band according to the appropriate CEPT - Recommendations.

All calculations (taking into account the predicted interference field strength and the calculated permissible interference field strength) are performed on a point to point basis. Therefor the data of the transmitter and the receiver are required as input values. To run this program a topographical database is required. In addition this program performs calculations on borderlines, preferential-lines and cross border ranges. For these calculations, data of the borderlines and preferential-lines are required, too. The preferential-lines can be calculated using the original borderline-data and the BORDER program which is available from the German Administration. If a morphological database is available, the detection of the distance over sea is done automatically.

Because the HCM program is only a subroutine, a surrounding program is required to run this software. Although it wasn’t the task of TWG HCM, a simple surrounding program is provided. This surrounding program is called HCMMS_V6.EXE. It serves as an interface to the HCM-subroutine, passing input data (entered by keyboard or stored in a file) to the subroutine and formatting the output to present it on the screen or write it to a text file.

2. Installation:

2.1 Minimum system and software requirements

To run the program, you need the following:

·	A PC with a 486/66 MHz or higher processor (recommended) running either the

Windows NT (Version 4) or Windows 95 or Windows 98 or Windows 2000 operating system.

- Sufficient memory, as follows:

- On Windows 95, 98 x86 systems, 16 megabytes (MB) of available memory (32MB or

- more recommended).

- On Windows NT or 2000 x86 systems, 24 megabytes (MB) of available memory (32MB

- or more recommended).

- A hard disk with enough disk space to install the program, terrain- and borderline data

- (program 1 MB, borderline data 1 MB, terrain data depending on the coverage area).

- A SVGA monitor (set up to 600 x 800 pixels or more).

- ·	A 3 ½  inch diskette device (1.44 MB).

- A mouse or compatible pointing device.

- Windows 95 / 98 / 2000 / Windows NT fonts installed on your system (‘MS San Serif 10’ and ‘Courier New 10’).

- 2.2 Required program files:

- You need the HCMMS_V6.EXE in a directory or sub-directory of your choice on your PC.

- In addition, you need ‘DRIVE.DAT’ in the working directory of the HCMMS_V6 program (normally the same directory, HCMMS_V6.EXE is located in).

- DRIVE.DAT contains one character, the drive letter where your terrain - and borderline data (and morphological data) is stored and a CR /LF. You can create this file using a text editor (NOTEPAD, WRITE etc.). Please enter the letter of the drive where your terrain- and line data is located (in uppercase) and the Enter-key. Store this file with the name DRIVE.DAT in the directory where HCMMS_V6.EXE is in.

- Note: Using DRIVE.DAT, you need (top level) directories named TOPO, BORDER (and MORPHO)!

- Please use the Explorer. Point to the HCMMS_V6.EXE, push the right mouse button and drag the icon to the desktop, select ‘Create shortcut(s) Here’.

- 2.2.1 Running the program

- To run the program, double-click the shortcut (icon).

- Note: If you use file input (Chapter 7), it is possible, to queue as many calculations as you like in one input file.

- 2.3 Required data files:

- The program requires the terrain data files as described in the HCM documentation and (border-) lines data files described in the HCM documentation. As described in the documentation, the directory structure of these files is fixed. If you use DRIVE.DAT, the only variable part of the full path is the drive. Terrain data is stored in directory: drive:\TOPO\..., line data is stored in directory: drive:\BORDER (and morphological data is stored in directory: drive:\MORPHO). To determine, on which drive your terrain- and border line data (and morphological data) is stored, a file DRIVE.DAT is required in the working directory of the program (see 2.2

- 3. Calculation modes:

- The program is developed for the VA01. But in addition, for non VA01 calculations, it is possible to supersede settings according VA01 by supplying input values for a lot of parameters.

- The HCM program gives you the choice of 18 different calculation-modes. These choices are called CMODE and numbered from - 8 to + 9.

- -	Negative CMODE - codes are for (border-) line-calculations. Transmitter-data has to be supplied and appropriate (border-) line should be selected.

- -	Zero and all positive CMODE-codes are for point to point calculations.

- There are 5 calculation modes according the VA01 and 13 modes covered by CEPT - Recommendations..

- 3.1 Calculation modes defined in the VA01:

- 3.1.1	CMODE 0:	VA01 calculations

- This calculation mode is used for evaluation of the interference situation between two stations in accordance with Annex 5 of the VA01.

- 3.1.2 CMODE -1: (Border-) line calculation (h2 = 10m)

- This calculation mode is used for evaluating the coordination situation of a planned station. Field strength calculations will be performed on lines. These calculations are on borderlines, x-km lines (preferential -lines) or cross-border-range-calculations.

- 3.1.3 CMODE 7: VA01 measurement calculation (50% time)

- This calculation mode is used for checking field strength measurements as described in Annex 7 of the VA01.

- 3.2 Calculation modes defined by CEPT - recommendations:

- 3.2.1 CMODE -2: (Border-) line calculation GSM (19 dBµV/m, h2 = 3m)

- This calculation mode is used for evaluating the coordination situation of a GSM base station in accordance with the CEPT Recommendation T/R 20-08. Field strength calculations will be performed on the border- or the 15 km line.

- 3.2.2 CMODE -3: Border line calculation ERMES (12 dBµV/m)

- This calculation mode is used for evaluating the coordination situation of a ERMES base station in accordance with the CEPT Recommendation T/R 25-07 in case that the introduction shall take place within 1 year of the concerned Administrations being informed (category a), or within 2 years (category b) or to countries where the channel is used by other landmobile applications than ERMES (category c). Field strength calculations will be performed on the border line.

- 3.2.3 CMODE -4: Border line calculation ERMES (32 dBµV/m)

- This calculation mode is used for evaluating the coordination situation of a ERMES base station in accordance with the CEPT Recommendation T/R 25-07 in case that non preferential frequencies are used (category c). Field strength calculations will be performed on the border line.

- 3.2.4 CMODE -5: Border line calculation ERMES (52 dBµV/m)

- This calculation mode is used for evaluating the co-ordination situation of a ERMES base station in accordance with the CEPT Recommendation T/R 25-07 in case that the introduction shall take place 1 year after the concerned Administrations have been informed (category a), or 2 years after they have been informed (category b). Field strength calculations will be performed on the border line.

- 3.2.5 CMODE -6: (Border-) line calculation DCS (25 dBµV/m)

- This calculation mode is used for evaluating the co-ordination situation of a DCS-1800 base station in accordance with the CEPT Recommendation T/R 22-07. Field strength calculations will be performed on the border- or on the 15 km line.

- 3.2.6 CMODE -7: (Border-) line calculation for the Emergency / Security Services

- band 380 - 400 MHz (18 dBµV/m)

- This calculation mode is used for evaluating the co-ordination situation in this frequency band for the Emergency / Security Services. Field strength calculations will be performed on the border- or on the 50 km line.

- 3.2.7 CMODE -8: (Border-) line calculation for the UMTS/IMT2000 Services

- This calculation mode is used for evaluating the co-ordination situation in this frequency band for the UMTS/IMT2000 Services. Field strength calculations will be performed on the border- or on the x km line.

- 3.2.8 CMODE 1: GSM <-> GSM

- This calculation mode is used for evaluating the interference situation between two GSM stations in accordance with the CEPT Recommendation TR 20-08 (not using the simplified sum interference algorithm as described in Annex 2 of this Recommendation and needed for co-ordination calculation).

- 3.2.9 CMODE 2: GSM <-> TACS

- This calculation mode is used for evaluating the interference situation between a GSM station and a TACS in accordance with the CEPT Recommendation TR 20-08 (not using the simplified sum interference algorithm as described in Annex 2 of this Recommendation and needed for co-ordination calculation).

- 3.2.10 CMODE 3: GSM <-> NMT

- This calculation mode is used for evaluating the interference situation between a GSM station and a NMT in accordance with the CEPT Recommendation TR 20-08 (not using the simplified sum interference algorithm as described in Annex 2 of this Recommendation and needed for coordination calculation).

- 3.2.11 CMODE 4: ERMES <-> ERMES (32dBµV/m)

- This calculation mode is used for evaluating the interference situation between ERMES FB and ML stations in accordance with the CEPT Recommendation TR 25-07.

- 3.2.12 CMODE 5: DCS ML <-> DCS FB (38dBµV/m)

- This calculation mode is used for evaluating the interference situation between a DCS-1800 mobile station and a DCS-1800 base station in accordance with the CEPT Recommendation TR 22-07 not using the simplified sum interference algorithm as described in Annex 2 of this Recommendation and needed for co-ordination calculation.

- 3.2.13 CMODE 6: DCS FB <-> DCS ML (42dBµV/m)

- This calculation mode is used for evaluating the interference situation between a DCS-1800 base station and a DCS-1800 mobile station in accordance with the CEPT Recommendation TR 22-07 not using the simplified sum interference algorithm as described in Annex 2 of this Recommendation.

- 3.2.14 CMODE 8: Calculation for the Emergency / Security Services

- band 380 - 400 MHz (18 dBµV/m)

- This calculation mode is used for evaluating the interference situation in this frequency band for the Emergency / Security Services.

- 3.2.15 CMODE 9: Calculation for the UMTS/IMT2000

- This calculation mode is used for evaluating the interference situation in this frequency band for the UMTS/IMT2000 Services.

- 4. Application of cases listed in Annex 5 of the VA01:

- Annex 5 of the VA01 gives descriptions of the different methods for calculating field strength. All methods (free space, knife edge, DEYGOUT, CCIR) are covered by the HCM program. Knife edge- and DEYGOUT method describes the propagation by diffraction and therefore HCM decided to name both methods as DEYGOUT method. The different cases in Annex 5 of the VA01 described as follows:

- 4.1 Base station / Fixed station (FB) Þ Base station / Fixed station (FB)

- This case is fully covered by the HCM program. (see 2.1 of Annex 5)

- 4.2 FB Þ Mobile station (ML)

- This case is fully covered by the HCM program.

- 4.3 ML Þ ML

- This case is fully covered by the HCM program.

- 4.4 ML Þ FB

- This case is fully covered by the HCM program.

- -	the effective transmitter antenna height is set to 3 m in the HCM program, if the input value of radius of service area is > 0.

- -	the effective transmitter antenna height is calculated in the HCM program, if the input value of radius of service area = 0.

- Rem.: 	Clause 3.1 of Annex 5 of the VA01 stated, that the calculation of the effective antenna height has to be performed only for FBs.

- 4.5 ML in particular place

- This case is covered in the HCM program by setting the input value of radius of service area equal to zero (2.5 of Annex 5 of the VA01).

- 4.6 Effective antenna height

- In clause 3.1 of Annex 5 of VA01, effective antenna heights are only foreseen for base stations. For this reason, the program use the input value of radius of service area. If this value is zero, the effective antenna height is calculated. If this value is greater than zero, the effective antenna height is set to 3 m.

- 4.7 Terrain irregularity Dh

- In clause 3.2 of Annex 5 of VA01, the calculation of Dh is described. Therefore, the HCM program takes into account this correction factor. If you which to perform non VA01 calculations without the corrections according Dh, you have to:

- -	write the results to a text file (VIENNA.TXT),

- -	find the correction factor according Dh in this text,

- -	modify the result according this correction factor.

- 4.8 Correcting factor for frequencies in the 900 MHz band

- In clause 3.3 of Annex 5 of the VA01, for frequencies in range 862 - 960 MHz the interference field strength has to be corrected by - 2 dB. The HCM program is in line with clause 3.3 of the VA01.

- 4.9 Terrain clearance angle

- In clause 3.4 of Annex 5 of VA01, the calculation of the correction factor for the terrain clearance angle is described. Therefore this correction factor is included in the program. If you like to perform non VA01 calculations without this correction factor, proceed as described in 4.7 of this User Guide.

- 4.10 Antenna diagram

- As stated in clause 3.5 of the Annex 5 of VA01, corrections according antenna diagrams are taken into account in the HCM program.

- 4.11 Receiver antenna height

- In the HCM program, the correction of the receiver antenna height is in accordance with 3.6 of Annex 5 of the VA01. For some special calculation modes, the receiver antenna height is set as defined in Table 1.1 of the description of module 1 of the HCM program.

- 4.12 Mixed path propagation

- In the HCM program, the mixed path propagation is in accordance with 3.7 of Annex 5 of the VA01. If no morphological database is available, the distance over sea has to be provided as an input value. If a morphological database is available, the distance over sea is calculated.

- 5. Input values:

- - Mode of calculation:	Mandatory, (chapter 3 of User Guide)

- - Transmitting frequency:	Mandatory

- - Geographical co-ordinates of the transmitter:	Mandatory (including seconds).

- - Height of transmitter above sea level:	Optionally. If missing, height is taken

- from terrain database.

- - Transmitter antenna height:	Mandatory

- - Type of antenna horizontal (Tx):	Mandatory; coding see VA01

- - Type of antenna vertical (Tx):	Mandatory; coding see VA01

- - Azimuth of maximum radiation Tx:	Mandatory. If horiz. Tx antenna is

- ‘000ND00’, value is not used.

- - Elevation angle of main radiation Tx:	Mandatory. . If verti. Tx antenna is

- ‘000ND00’, value is not used.

- - Maximum radiated power:	Mandatory; in dBW.

- - Type of Tx antenna:	Mandatory; "E" or "I"

- - Channel occupation:	Mandatory. "0" or "1". For some

- CMODES not used (default = 0).

- - Radius of the service area of the transmitter:	Mandatory. If 0, effective antenna height

- of Tx is calculated, if > 0, eff. antenna

- height is set to 3 m (for mobiles).

- - Designation of emission Tx:	Mandatory for some point to point

- calculations, value not used for (border-)

- line calculations.

- - Distance over sea:	Optionally; in km. If missing, the

- distance over sea is calculated if

- morphological data is available.

- - Sea temperature:	Mandatory; cold or warm ("C" or "W").

- C = North sea, W = Mediterranean sea.

- - Switch off DEYGOUT method:	Mandatory for point to point

- calculations, value not used for (border-)

- line calculations (= OFF).

- - Reception frequency:	Mandatory for point to point

- calculations, value not used for (border-)

- line calculations.

- - Geographical co-ordinates of Rx;	Mandatory for point to point

- calculations, value not used for (border-)

- line calculations.

- - Height of receiver above sea level:	Optionally. If missing, height is taken

- from terrain database. Value not used for

- (border-) line calculations.

- - Receiver antenna height:	Mandatory for some point to point

- calculations, value not used for (border-)

- line calculations.

- - Type of antenna horizontal (Rx):	Mandatory for point to point

- calculations, value not used for (border-)

- line calculations. Coding see VA01

- - Type of antenna vertical (Rx): 	Mandatory for point to point

- calculations, value not used for (border-)

- line calculations. Coding see VA01

- - Azimuth of maximum gain of Rx antenna:	Mandatory for point to point

- calculations, value is not used for

- (border-) line calculations or if horiz. Rx

- antenna is ‘000ND00’.

- - Elevation angle of main gain of Rx antenna:	Mandatory for point to point

- calculations, value is not used for

- (border-) line calculations or if verti. Rx

- antenna is ‘000ND00’.

- - Gain of Rx-antenna:	Mandatory for point to point

- calculations, value is not used for

- (border-) line calculations.

- - Type of Rx antenna gain:	Mandatory for point to point

- calculations ("E" or "I"), value is not

- used for (border-) line calculations.

- - Designation of emission Rx:	Mandatory for point to point

- calculations, value is not used for

- (border-) line calculations.

- - Depolarisation loss:	Mandatory for point to point

- calculations, value is not used for

- (border-) line calculations.

- - Radius of the service area of the receiver:	Mandatory. If 0, effective antenna height

- of Rx is calculated, if > 0, eff. antenna

- height is set to 3 m (for mobiles).

- - Input-value: permissible field strength:	Optionally, if missing, value is taken

- from table in Annex 1 of VA01.

- - Input value of correction factor

- according frequency difference:	Optionally. Not used for (border-) line

- calculations. If missing, value is

- calculated.

- - Land to calculate from (code of country):	Mandatory

- See Preface to the IFL.

- - Land to calculate to (code of country): 	Mandatory

- See Preface to the IFL.

- - Distance to borderline: 	Mandatory for (border-) line

- calculations. Not used for point to point

- calculations.

- negative values = Cross-border range

- 0 = calculation on borderlines

- positive value x= calculation on x km

- line.

- - Input value of „Maximum cross

- border range of harmful interference“:	Optionally for (border-) line

- calculations. If missing, value is taken

- from table in Annex 1 of VA01.

- 6. Output values:

- - Error value (see list), 0 = no error.

- - Distance between point A and B in km (only point to point calculations).

- - Height of Tx above sea level (terr. database) in m.

- - Height of Rx above sea level (terr. database) (only point to point calculations) in m.

- - Information (fields) (see list).

- - Tx antenna info (if true, Tx-antenna-type = "999XX99")

- - Rx antenna info (if true, Rx-antenna-type = "999XX99") (only point to point calculations).

- - Loss [dB] main obstacle (DEYGOUT) (only point to point calculations).

- - Loss [dB] second obstacle (DEYGOUT) (only point to point calculations).

- - Eff. antenna height [m] (CCIR) (Tx) (only point to point calculations).

- - Eff. antenna height [m] (CCIR) (Rx) (only point to point calculations).

- - Total eff. antenna height [m] (CCIR) (only point to point calculations).

- - Delta h [m] (CCIR) (only point to point calculations).

- - Correction factor k delta h [dB] (CCIR) (only point to point calculations).

- - Power [dBW] in direction of receiver (only point to point calculations).

- - Free space field strength [dBµV/m] (only point to point calculations).

- - Field strength [dBµV/m] (CCIR) land (only point to point calculations).

- - Field strength [dBµV/m] (CCIR) sea (only point to point calculations).

- - Correction according antenna attenuation [dB] (Tx) (only point to point calculations).

- - Correction according antenna type [E or I] [dB] (Tx) (only point to point calculations).

- - Azimuth (Tx) [degree]

- - Elevation (Tx) [degree]

- - Direction to Rx [degree] (only point to point calculations).

- - Angle vertical Tx to Rx [degree] (only point to point calculations).

- - Difference angle vertical (Tx-Rx - elev.) [degree] only point to point calculations).

- - Difference angle horizontal (Tx-Rx - azim. Tx) [degree] (only point to point calculations).

- - Angle vertical Rx to Tx [degree] (only point to point calculations).

- - Difference angle horizontal (Rx-Tx - azim. Rx) [degree] (only point to point calculations).

- - Difference angle vertical (Rx-Tx - elev. Rx) [degree] (only point to point calculations).

- - Azimuth (Rx) [degree] (only point to point calculations).

- - Elevation (Rx) [degree] (only point to point calculations).

- - Direction to Tx [degree] (only point to point calculations).

- - Clearance angle [degree] (Tx) (only point to point calculations).

- - Correction factor according clearance angle [dB] (Tx) (only point to point calculations).

- - Clearance angle [degree] (Rx) (only point to point calculations).

- - Correction factor according clearance angle [dB] (Rx) (only point to point calculations).

- - Landfigure

- - Seafigure

- - Distance over sea [km] (calculated or from input value)

- - Correction according antenna attenuation (Rx) [dB] (only point to point calculations).

- - Correction according antenna type [E or I] (Rx) [dB] (only point to point calculations).

- - Delta frequency [kHz] (only point to point calculations).

- - Correction factor according delta frequency [dB] (only point to point calculations).

- - Channel spacing of Rx [kHz] (only point to point calculations).

- - Channel spacing of Tx [kHz] (only point to point calculations).

- - Field strength (DEYGOUT or CCIR) [dBµV/m] (only point to point calculations).

- - Calculated field strength [dBµV/m] (only point to point calculations).

- - Permissible field strength [dBµV/m]

- - Permissible field strength of table in Annex 1 of VA01 [dBµV/m]

- - Max. range of harmful interference [km] (only point to point calculations).

- - E.R.P. of reference transmitter [dBW] (only point to point calculations).

- - Protection margin (EP - EC) [dB]

- - Maximum field strength at (border-)line [dBµV/m] (only (border-) line calculations).

- - Co-ordinates of (border-) line - point (only (border-) line calculations).

- - Co-ordinates of (calculated) Tx - point (differs from input value if Tx is a mobile).

- - Co-ordinates of (calculated) Rx - point (differs from input value if Rx is a mobile).

- - Distance to (border-) line - point [km] (only (border-) line calculations).

- - Direction to (border-) line - point [degree] (only (border-) line calculations).

- - Eff. antenna heights in 10 degree steps without antenna height [m] (only set in case of (border-) line - calculations)

- Error values :

- Negative values and values from 1 to 999 are reserved for the subroutine to access the terrain – or morphological data (Error 36 = no terrain data available).

- 1000 = Distance is greater than 1000 km, calculations not possible

- 1002 = Error in input value transmitter or receiver height

- 1003 = Error in input data of depolarization loss.

- 1004 = Error in input data of correction factor according frequency difference.

- 1005 = Error in antenna height

- 1006 = Error in elevation (Tx)

- 1007 = Error in antenna type

- 1008 = Error in azimuth (Tx)

- 1009 = error in type of antenna (Tx) (“E“ or “I“)

- 1010 = Error in power

- 1011 = Error in radius of service area

- 1013 = Channel spacing outside definition range (Rx)!

- 1014 = Channel spacing outside definition range (Tx)!

- 1015 = Error in azimuth (Rx)

- 1016 = Error in elevation (Rx)

- 1017 = Error in type of antenna ("E" or "I") (Rx)

- 1018 = Error in gain of antenna

- 1019 = Error in input data of permissible field strength

- 1021 = Error in geographical coordinates

- 1022 = Error in transmitting frequency

- 1023 = Error in reception frequency

- 1024 = Mode of calculation is not available

- 1025 = Error in input value of border line distance.

- 1026 = Selected (border-) line data not available.

- 1027 = Error in (border-) line data.

- 1028 = Error in (border-) line coordinates.

- 1029 = Error in input value of distance over sea.

- 1030 = Error in coordinates (Tx = Rx, -> distance = 0.0 km).

- 1031 = Error in input value „Maximum crossborder range of harmful interference“.

- 1032 = Error in service area Rx.

- 1033 = The xxx.ALL borderline file for the Tx is not available.

- 1034 = The xxx.ALL borderline file for the Rx is not available.

- Possible info values :

- No height of transmitter is given -> terrain data is used.

- Height of transmitter differs from terrain data.

- No height of receiver is given -> terrain data is used.

- Height of receiver differs from terrain data.

- Free space field strength used, because distance < 1 km

- Free space field strength used, because 1st Fresnel zone is free

- Only one obstacle. DEYGOUT is used.

- More than 2 obstacles. CCIR is used.

- 2nd obstacle between M1 and Rx

- 2nd obstacle between Tx and M1

- 1st obstacle too close to Rx, 2nd obstacle not calculated

- 1st obstacle too close to Tx, 2nd obstacle not calculated

- Two obstacles. DEYGOUT is used.

- Transmitter height differs more than 10%, calculated values may be (extremely) wrong!

- Receiver height differs more than 10%, calculated values may be (extremely) wrong!

- Distance over sea is greater than total distance. Distance between Tx and Rx is used!

- CCIR is used because DEYGOUT is switched off.

- The transmitter frequency is outside the range of table in Annex 1. The max. crossborder range, the permissible field strength and the ERP of the reference transmitter are set to 0.

- Input value used for permissible field strength!

- Input value used for correction factor according frequency difference!

- Frequency difference outside definition range, 82 dB is used.

- Input value used for „Maximum crossborder range of harmful interference“.

- File ‘BORDER.XXX’ in the BORDER directory is not found or the contents is wrong.

- The field strength is set to 999.9, because of service area overlapping.

- Tx antenna type is ‘999XX99’.

- Rx antenna type is ‘999XX99’.

- 7. Using text files for the input:

- For simple surrounding programs (e.g. written in MS ACCESS), it is possible to select all input data and write it to a text file. Then the HCMMS_V6 program can be run using the ‘read input from file’ option. In this case, no additional input is requested and the output is written to a text file ‘HCM.TXT’. The format of the input text file differs according to the CMODE. Examples are shown below.

- Note: It is possible to queue as many calculation input data as you like. Simply add more lines to the input text file.

- The examples can be used as reference for the format of data not listed in Annex 2 of the VA01.

- Example 1 for a data file containing all input for a borderline calculation.

- Data	Description / Format

- (not part of data!)

| - | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | mode of calculation /N2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 6 | E | 2 | 0 | 0 | 0 | 5 | 1 | N | 0 | 0 | 0 | 0 |  | Tx co-ordinates / C15 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Tx height above sea level / NC4 |
| 0 | 0 | 0 | N | D | 0 | 0 |  |  |  |  |  |  |  |  |  | Tx antenna type horizontal / C7 |
| 0 | 0 | 0 | N | D | 0 | 0 |  |  |  |  |  |  |  |  |  | Tx antenna type vertical / C7 |
|  |  | 0 | . | 0 |  |  |  |  |  |  |  |  |  |  |  | azimuth / D5.1 |
|  |  | 0 | . | 0 |  |  |  |  |  |  |  |  |  |  |  | elevation / D5.1 |
|  |  | 2 | 0 |  |  |  |  |  |  |  |  |  |  |  |  | Tx antenna height / N4 |
|  |  | 1 | 0 | . | 0 |  |  |  |  |  |  |  |  |  |  | radiated power / D6.1 |
| E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Tx type of antenna (E or I) / C1 |
| 0 | 0 | 9 | 5 | 4 | . | 4 | 0 | 0 | 0 | 0 | M |  |  |  |  | Tx frequency / D11.5 + C1 |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | channel occupation / N1 |
| C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | sea temperature (C or W) / C1 |
|  |  | 0 | . | 0 |  |  |  |  |  |  |  |  |  |  |  | distance over sea / DC5.1 |
|  |  |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  | radius of service area / N5 |
|  |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  | distance to borderline (km) / N4 |
| D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | land of Tx (IFL - code) / C3 |
| H | O | L |  |  |  |  |  |  |  |  |  |  |  |  |  | land to calculate to / C3 |
|  | 1 | 9 | . | 0 |  |  |  |  |  |  |  |  |  |  |  | perm. field str. (input) / DC5.1 |
| 1 | 0 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  | max. cbr (input) / C3 |

Format description:

Nx = Number, x = number of digits, right justified.

Cx = Character (letters or numbers), x  = maximum number of characters, left justified.

Dx.y = Decimal numbers, x = total length (including the decimal point), y = number of digits

after the decimal point.

NCx = Same as Nx, but it is possible to leave this field empty (blanks).

DCx.y = Same as Dx.y, but it is possible to leave this field empty (blanks).

Example 2 for a data file containing all input for a normal calculation.

Data	Description / Format

(not part of data!)

|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | mode of calculation / N2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 6 | E | 2 | 2 | 3 | 3 | 5 | 1 | N | 1 | 5 | 2 | 5 |  | Tx co-ordinates / C15 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Tx height above sea level / NC4 |
| 0 | 0 | 0 | N | D | 0 | 0 |  |  |  |  |  |  |  |  |  | Tx antenna type horizontal / C7 |
| 0 | 0 | 0 | N | D | 0 | 0 |  |  |  |  |  |  |  |  |  | Tx antenna type vertical / C7 |
|  |  | 0 | . | 0 |  |  |  |  |  |  |  |  |  |  |  | azimuth of Tx antenna / D5.1 |
|  |  | 0 | . | 0 |  |  |  |  |  |  |  |  |  |  |  | elevation of Tx antenna / D5.1 |
|  |  | 1 | 0 |  |  |  |  |  |  |  |  |  |  |  |  | Tx antenna height / N4 |
|  |  | 1 | 0 | . | 0 |  |  |  |  |  |  |  |  |  |  | radiated power / D6.1 |
| E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Tx type of antenna (E or I) / C1 |
| 0 | 0 | 1 | 4 | 7 | . | 7 | 7 | 0 | 0 | 0 | M |  |  |  |  | Tx frequency / D11.5 + C1 |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | channel occupation / N1 |
| C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | sea temperature (C or W) / C1 |
|  |  | 0 | . | 0 |  |  |  |  |  |  |  |  |  |  |  | distance over sea / DC5.1 |
|  |  |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  | radius of Tx service area / N5 |
| Y |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DEYGOUT allowed (Y or N) / C1 |
| 0 | 0 | 6 | E | 0 | 0 | 0 | 0 | 5 | 1 | N | 0 | 0 | 0 | 0 |  | Rx coordinates / C15 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rx height above sea level / NC4 |
|  |  | 1 | 0 |  |  |  |  |  |  |  |  |  |  |  |  | Rx antenna height / N4 |
| D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Country code of Tx / C3 |
| H | O | L |  |  |  |  |  |  |  |  |  |  |  |  |  | County code of Rx / C3 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | perm. field str. (input) / DC5.1 |
| 0 | 0 | 1 | 4 | 7 | . | 7 | 7 | 0 | 0 | 0 | M |  |  |  |  | Rx frequency / D11.5 + C1 |
| 1 | 4 | K | 0 |  |  |  |  |  |  |  |  |  |  |  |  | Tx designation of emission / C9 |
| 1 | 4 | K | 0 |  |  |  |  |  |  |  |  |  |  |  |  | Rx designation of emission / C9 |
| 0 | 0 | 0 | N | D | 0 | 0 |  |  |  |  |  |  |  |  |  | Rx antenna type horizontal / C7 |
| 0 | 0 | 0 | N | D | 0 | 0 |  |  |  |  |  |  |  |  |  | Rx antenna type vertical / C7 |
|  |  | 0 | . | 0 |  |  |  |  |  |  |  |  |  |  |  | azimuth of Tx antenna / D5.1 |
|  |  | 0 | . | 0 |  |  |  |  |  |  |  |  |  |  |  | elevation of Rx antenna / D5.1 |
| E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rx type of antenna (E or I) / C1 |
| 0 | 0 | . | 0 |  |  |  |  |  |  |  |  |  |  |  |  | gain of Rx antenna / D4.1 |
| 0 | 0 | . | 0 |  |  |  |  |  |  |  |  |  |  |  |  | depolarization loss / D4.1 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | corr. factor acc. freq. diff. / NC2 |
|  |  |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  | radius of Rx service area / N5 |

8. Calculations on lines:

There are different choices to calculate on lines:

- Calculations on borderlines

- Calculations on x-km-lines and

- Calculations on the cross border range

- A frequently asked question is: In which case do I have to calculate on a borderline (an x-km-line, the cross border range)?

- The answer is: It depends on the situation.

- For normal Vienna Agreement:

- First calculations to coordinated stations are required. In cases, where no coordinated station is disturbed, further calculations are required: First calculations on the borderlines are performed. If the calculated field strength on a borderline is less than the permissible field strength, no coordination procedure with this country is required. If the calculated field strength is higher, coordination is necessary and an addition calculation on the cross border range is required. If the calculated field strength on the cross border range is higher than the permissible field strength, the coordination request can be rejected.

- Note: For normal Vienna Agreement no calculations on x-km-lines are performed!

- In case of a preferential frequency agreement:

- If there is a preferential frequency agreement, first step is to calculate to coordinated stations (if it is written in the agreement, that coordinated stations have to be protected). Second step is to calculate on borderlines to determine the involved countries (relationship between two, three or four countries). Taken into account the involved countries, the frequency and the agreement, the question whether it is a preferential frequency of the own country or not can be answered. In case of a preferential frequency of the own country and a field strength at the borderline higher than the permissible field strength, an additional calculation on the x-km-line is required.

- Coordination request is required in case of no own preferential frequency and the field strength at the borderline higher than the permissible field strength or

- if there is an own preferential frequency and the field strength at the x-km-line higher than the permissible field strength.

- Information is required in case of an own preferential frequency, the field strength on the borderline higher and the field strength on the x-km-line lower than the permissible field strength.

- Note: For preferential frequency agreements no calculations on the cross border range is performed!

- In general: All bi- or multilateral agreements (preferential frequency agreements, lattice plans etc.) overrule the Vienna Agreement.

- 9. Questions and answers:

- Elevation positive or negative ?

- If the antenna is pointing towards the earth, the elevation angle is negative;

- if the antenna is pointing towards the sky, the elevation angle is positive.

- How to select x-km line - calculations?

- At the 2nd page of the input - dialogue (Distance to the borderline),

- type in the value x (e.g. 15 for the 15 km - line).

- How to select cross-border-range - calculations?

- At the 2nd page of the input - dialogue (Distance to the borderline),

- type in a negative value (e.g. -1).

- In case where a preferential frequency plan is available:

- Calculate on the x-km line into direction of country A, but the x-km point is inside country B. Has country A or B or both to be coordinated with?

- The question, which country is to coordinate with, depends only on the field strength

- at the borderline of this country. If this field strength is higher than the permissible

- field strength, a preferential frequency has to be used or coordination is required.

- If the field strength at the x-km-line is higher than the permissible field strength,

- a coordination request can be rejected from those countries, independent whether

- the x-km-point is in or outside those countries.

- Example for x-km-line calculations

country D

country C

country B

countryA

15 km

12 km

*

studied transmitter

To determine in which coordination situation the transmitter * is in, calculations has to be performed on the border-line of the countries B, C and D. Depending on the result of these calculations a preferential frequency is needed in the two-, three- or four country case. Calculation on the X-km line is performed X km beyond the border into the neighbouring country; it is of no importance if the neighbouring country is smaller than this distance x. (see drawing below).

- In case where a preferential frequency plan is not available:

- Calculate the cross-border-range to country A, but cross-border-point is in country B. Has country A or B or both to be coordinated with?

- The question, which country is to coordinate with, depends only on the field strength

at the borderline of this country. If this field strength is higher than the permissible

field strength, coordination is required.

If the field strength at the cross-border-range is higher than the permissible field

strength, the coordination request can be rejected from those countries, independent

whether the point is in or outside those countries.