Storage format of the height data

The height data used by the HCM programs (Mobile Service and Fixed Service) may come from different sources.

Some data may come from military source (Digital Terrain Elevation Data (DTED) Level 1 Coverage), some data are may come from national geographic institutes.

A fallback possibility to get terrain height data is to use GTOPO30 data. This data is free available at the Internet (http://edcwww.cr.usgs.gov/landdaac/gtopo30/gtopo30.html), but it only has a resolution of 30 seconds (approximately 1 km). GTOPO30 data converted to the HCM format are available at the internet site of the Vienna Agreement (http://va.bmwi.bund.de).

The HCM terrain height data has a resolution of 3 seconds in the North - South direction and 3 or 6 seconds in the East – West direction (depending on the latitude). If the latitude is less than 50 degrees (North or South), the resolution is 3 seconds, if the latitude is greater or equal to 50 degrees, the resolution is 6 seconds.

All source data has to be converted to WGS84 format and to the above mentioned resolution. Data from different sources needs to be combined to a common database covering all Vienna Agreement Signatory countries (plus an additional range of approximately 100 km).

The storage format of this data is:

- All elevation data consists of 2 Bytes Integer-values (Fixed Binary integers). If the elevation value is negative, first the MSB (Bit #15) has to be cleared and second the complement has to be built to get the correct elevation value.

- Terrain data of a 5 x 5 minute square (approximately 9 x 5 km) is combined in one data-record. Strips in the North and East are added to the data inside the square. The reason is: If you want to have the elevation of a given point, in most cases this point is located between 4 points in the grid of the stored data. To get the correct elevation, you have to interpolate between these 4 points. The western and southern grid-points are always present (example: wanted point 8 degrees 0 minute 1 second → record 8 degrees 0 minute to 5 minutes is read; so the western grid-point (= 8 degrees 0 minute 0 second) is present. In the case where the wanted point is for example 8 degrees 4 minutes 59 seconds, the eastern grid-point is 8 degrees 5 minutes 0 second. Normally this point is not inside the read record, but part of another record (8 degrees 5 minutes to 10 minutes). To prevent the program from reading an additional record, the strip 5 minutes 0 second is added to the record 0 minute to 5 minutes. For the same reason, a strip in the North is also added. It is therefore possible to get the right elevation of a wanted point reading only one record of data.

- 12 x 12 records (=144 records;=1 x 1 degree) are stored in one file.

- The filename is (example): E007N50.63E

| where | E007 | = | longitude of the South-West corner, |
| --- | --- | --- | --- |
|  | N50 | = | latitude of the South-West corner, |
|  | 63 | = | resolution in seconds longitude (6) and latitude (3) and |
|  | E | = | elevation data (M for morphological data) |

- Position of records inside the file:

North

|  | 133 | 134 | 135 | 136 | 137 | 138 | 139 | 140 | 141 | 142 | 143 | 144 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 121 |  |  |  |  |  |  |  |  |  |  | 132 |  |
|  | 109 |  |  |  |  |  |  |  |  |  |  | 120 |  |
|  | 97 |  |  |  |  |  |  |  |  |  |  | 108 |  |
|  | 85 |  |  |  |  |  |  |  |  |  |  | 96 |  |
|  | 73 |  |  |  |  |  |  |  |  |  |  | 84 |  |
| West | 61 |  |  |  |  |  |  |  |  |  |  | 72 | East |
|  | 49 |  |  |  |  |  |  |  |  |  |  | 60 |  |
|  | 37 |  |  |  |  |  |  |  |  |  |  | 48 |  |
|  | 25 |  |  |  |  |  |  |  |  |  |  | 36 |  |
|  | 13 |  |  |  |  |  |  |  |  |  |  | 24 |  |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |  |

South

- All files with the same longitude are stored in the same subdirectory. The name of the subdirectory is equal to the first four characters of the filename (example: E007).

- All subdirectories are stored in the (top level) directory TOPO. A valid filename with the full path therefore is:

C:\TOPO\E010\E010N45.33E

Note: Only the drive letter ‘C’ may be replaced by any other valid drive letter. The rest of the directory structure is fixed. This fixed directory structure is build in the HCM programs. Only the drive letter is an input value to the HCM programs. The rest of the filename / subdirectory name is calculated inside the HCM programs taking into account the co-ordinates.

- In Europe, a 5 minutes x 5 minutes square contains north of 50 degrees latitude 101 x 51 values = 5.151 values and south of 50 degrees latitude 101 x 101 values = 10.201 values. One value = 2 Bytes. The length of data records therefore is 10.302 or 20.402 Bytes.

- Record description:

Length: fixed, 10.302 or 20.402 Bytes

No carriage control!

- The elevation data inside the record is combined from East to West and from South to North.

Example:  (South of 50 degrees latitude, resolution in East-West-direction = 3 seconds; number of elevation data)

|  |  |  |  |  |  |  | North |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10101 |  | 10102 |  | 10103 |  |  |  |  |  | 10200 |  | 10201 |  |  |
|  |  | 10000 |  | 10001 |  | 10002 |  |  |  |  |  | 10099 |  | 10100 |  |
| West |  |  |  |  |  |  |  |  |  |  |  |  |  |  | East |
|  |  | 102 |  | 103 |  | 104 |  |  |  |  |  | 201 |  | 202 |  |
|  | 1 |  | 2 |  | 3 |  |  |  |  |  | 100 |  | 101 |  |  |

South